# Memorandum

To:	Clear Roads Technical Advisory Committee

From:	Keith Pilgrim

Subject:	Quarterly Project Update

Date:	7/10/2012

Project:	Determining the Toxicity of Deicing Materials: MnDOT Contract #99083

Project activities during this quarter included several discussions with Ron Write regarding toxicological endpoints and the estimation of product doses to be used in the range finding tests (attached).  Does were initially identified for the range finding tests and upon discussion with the committee, doses were adjusted.  Toxicity testing is scheduled for late July.  We have also been provided deicing products by the vendors identified in the table below.  A salt brine inhibitor in Category A2, Boost SB, is no longer provided by America West.

| Category | Product | Manufacturer |
| --- | --- | --- |
| Category I Magnesium Chloride | FreezGard CI Plus | North American Salt |
|  | Meltdown APEX with Shield AP | Envirotech |
| Category II Calcium Chloride | Calcium Chloride with Boost | America West |
|  | Road Guard Plus | Road Guard Plus |
| Category A1 Salt Brine Inhibitors | Watershed Cl | Rivertop Renewables |
| Category A2 Salt Brine Inhibitors | Boost SB | America West (NOT AVAILABLE) |
| Category A3 Salt Brine Inhibitors | Articlear Gold | North American Salt |
| Experimental Category | Potasium Acetate CF-7 | Cryotech |
|  | Alpine Ice Melt (K-Acetate) | Nachurs Alpine |
| Glycerol | Apogee | Envirotech |

Literature Review

A memorandum describing the findings of the literature review was provided to the committee for review.  This literature review will be incorporated into the final report.

| Doses Identified for the Range Finding Toxicity Tests: Category I: Magnesium Chloride |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Species | FreezGard concentration (g/l) |  |  |  |  |  |  |  |
|  | Dose 1 | Dose 2 | Dose 3 | Dose 4 |  |  |  |  |
| Fathead Minnow | 0.03 | 0.10 | 0.30 | 1.00 |  |  |  |  |
| C. dubia | 0.25 | 0.50 | 1.00 | 4.00 |  |  |  |  |
| Selenastrum | 1.00 | 2.00 | 8.00 | 20.00 |  |  |  |  |
| Category II: Calcium Chloride |  |  |  |  |  |  |  |  |
|  | Road Guard Plus concentration (g/l) |  |  |  |  |  |  |  |
| Species | Dose 1 | Dose 2 | Dose 3 | Dose 4 |  |  |  |  |
| Fathead Minnow | 1.00 | 3.00 | 6.00 | 15.00 |  |  |  |  |
| C. dubia | 0.25 | 0.50 | 1.50 | 6.00 |  |  |  |  |
| Selenastrum | 1.00 | 2.00 | 4.00 | 12.00 |  |  |  |  |
| Potassium Acetate |  |  |  |  |  |  |  |  |
|  | CF-7 concentration (g/l) |  |  |  |  |  |  |  |
| Species | Dose 1 | Dose 2 | Dose 3 | Dose 4 |  |  |  |  |
| Fathead Minnow | 0.05 | 0.20 | 0.50 | 2.00 |  |  |  |  |
| C. dubia | 0.01 | 0.05 | 0.20 | 2.00 |  |  |  |  |
| Selenastrum | 2 | 20 | 100 | 200 |  |  |  |  |
| Experimental Category: Glycerol |  |  |  |  |  |  |  |  |
|  | Apogee concentration (g/l) |  |  |  |  |  |  |  |
| Species | Dose 1 | Dose 2 | Dose 3 | Dose 4 |  |  |  |  |
| Fathead Minnow | 0.01 | 0.05 | 0.10 | 0.50 |  |  |  |  |
| C. dubia | 0.05 | 1.00 | 5.00 | 10.00 |  |  |  |  |
| Selenastrum | 0.05 | 1.00 | 5.00 | 10.00 |  |  |  |  |
| concentrations are for Apogee |  |  |  |  |  |  |  |  |
| Inhibitors Used with Sodium and Calcium Chloride |  |  |  |  |  |  |  |  |
| Species | Inhibitor A1 concentration (g/l) |  |  |  |  |  |  |  |
|  | Dose 1 | Dose 2 | Dose 3 | Dose 4 |  |  |  |  |
| Fathead Minnow | 0.50 | 3.00 | 6.00 | 12.00 |  |  |  |  |
| C. dubia | 0.005 | 0.10 | 1.00 | 3.00 |  |  |  |  |
| Selenastrum | 2.00 | 6.00 | 15.00 | 30.00 |  |  |  |  |